## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics Engg./EC

Subject: Power Electronics(4143)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the construction & characteristics of LASCR. |  |
| b) | Explain reverse recovery time & gate recovery time of SCR. |  |
| c) | Explain di/dt protection of a thyristor. |  |
| d) | Explain the need for connecting power diodes in series & parallel. |  |
| e) | Explain the application of triac as a switch. |  |
| f) | List applications of a cycloconverter & a chopper. |  |
| Q.No.2. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain transient characteristics of SCR. |  |
| b) | Explain UJT triggering method of SCR using neat diagram & waveforms. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain class ‘C’ commutation method. |  |
| b) | Explain VI characteristics of triac. |  |
| c) | Explain parallel operation of power diodes in detail. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of single phase half wave controlled rectifier with RL load. Also state the equation of output D.C voltage. |  |
| b) | Explain the operation of single phase full wave controlled rectifier with RL load with free wheeling diode. |  |
| c) | Explain the operation of 3 phase half wave rectifier. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of Mc-Murray inverter. |  |
| b) | Explain the operation of single thyristor chopper. |  |
| c) | Explain the operation of 16 2/3 Hz cycloconverter circuit. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of single phase motor starter using triac. |  |
| b) | Explain the operation of SCR controlled static regulator. |  |
| c) | Explain the block diagram operation of SMPS. |  |

***********