## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics Engg./EC/MED/E&EE

Subject: Circuits & Networks(4136)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define mesh, loop and node. |  |
| b) | Find equivalent resistance between A & B. Refer fig.(1). |  |
| c) | Find equivalent capacitance between C & D. Refer fig.(2). |  |
| d) | What is Active power and Reactive power in a series R-L circuit? |  |
| e) | Draw & label the graphical representation of RLC series resonance circuit. |  |
| f) | Explain the term ‘Image impedance’. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Kirchhoff’s Voltage law and Current law with a neat diagram. |  |
| b) | Find the ammeter current in fig.(3) by using loop analysis. |  |
| c) | Compute the network resistance measured between A & B fig.(4). |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of superposition theorem, find the current Iab in the circuit of fig.(5). |  |
| b) | Use Thevenin’s theorem to calculate the current flowing through 16 ohm resistance of fig.(6). |  |
| c) | Using nodal analysis, find current I in fig.(7). |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An ac series circuit has a resistance of 10, an inductance of 0.2H and a capacitance of 60F. Calculate: - The resonant frequency - The current at resonance - The power at resonance - Quality factor Given that the applied voltage is 200V. |  |
| b) | A coil of resistance 10 ohm and inductance 0.1 Henry is connected in series with a 150F capacitor across a supply of 200V,50Hz. Calculate i) Inductive reactance ii) Capacitive reactance iii) Impedance iv) the current v) the power factor vi) power in watts. |  |
| c) | Calculate load impedance that draws the maximum power from the source shown in fig.(8). | …..2/- |

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With the help of circuit diagram, waveforms & suitable expressions explain the operation of RC differentiator circuit. |  |
| b) | Derive expression for transient voltage across the capacitor in series RC circuit when it is connected to a DC source. Sketch the response. |  |
| c) | Obtain expression for current in the circuit when a 15H inductance coil in series with 10 ohm resistance is suddenly connected to a 20V dc supply. Calculate the current after 2 seconds. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a symmetrical T-network. Derive expression for its characteristic impedance (Z0) and short circuit impedance (Zsc). |  |
| b) | Explain the terms i) Characteristic impedance and ii) Propagation constant of symmetrical network. |  |
| c) | i)Design a symmetrical lattice attenuator to have a characteristic impedance of 500 ohm & provide an attenuation of 20dB. |  |
|  | ii) Design Bridged-T attenuator to provide an attenuation of 40dB and operate between two 600 ohm impedance. |  |

……3/-