## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Basic Instrumentation-II(S-4216)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | With the help of an example differentiate between Precision and Accuracy. |  |
| b) | State the ideal characteristics of opamp. Draw the circuit of a buffer amplifier using opamp. |  |
| c) | Explain the classification of data presentation elements. |  |
| d) | Draw the equivalent circuit of piezo-electric sensor. State two applications. |  |
| e) | Explain the coupling techniques used in Instrumentation. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare incremental encoders and absolute encoders. |  |
| b) | Explain the working principle and construction of an electrical tachometer. |  |
| c) | Explain the principle of working of a fibre optic transducer. How can a fibre optic transducer be used to measure level. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give one example each of a mechanical amplifier and hydraulic amplifier. Draw neat diagrams, label the input and output. State the relationship between input and output. |  |
| b) | What is the purpose of filters? List the different types of filters and sketch their frequency response. |  |
| c) | Compare ac and dc amplifiers. Which amplifier is suitable for conditioning the output of a thermocouple? Explain why. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of a moving coil mechanism with the help of a diagram. State its applications in Instrumentation. |  |
| b) | Draw the block diagram of a typical oscilloscope. Compare between the electrostatic and electromagnetic deflection system. |  |
| c) | Define Amplitude modulation. Draw the waveform of an amplitude modulated signal carrier and modulating signal. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following terms and their significance in Instrumentation: i) Linearity ii) Hysteresis iii) Resolution. |  |
| b) | Sketch and explain the response of a first order system to a unit step input. |  |
| c) | Explain the shielding and grounding techniques used in Instrumentation. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the terms response time, rise time and settling time. |  |
| b) | Write a note on servo recorder. |  |
| c) | Draw the circuit diagram, input and output waveforms of the following: i) Integrator ii) Differentiator. Also state the relationship between input and output. |  |

***********