## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Mechanical Engg.

Subject: Power Engineering-I(4037)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Steam tables and Mollier’s charts is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State and explain Gay-Lussac’s law. |  |
| b) | Define Enthalpy of steam. Write equation for enthalpy of superheated steam. |  |
| c) | State the uses of compressed air. |  |
| d) | State the reasons for modifying the Rankine cycle for operation of steam engine. |  |
| e) | Distinguish between “Specific heat at constant pressure” and “specific heat at constant volume”. |  |
| f) | What do you understand by the term “wet steam”? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The following data refers to a calorific value test of a fuel by means of a gas calorimeter Volume of gas used = 0.7 m3 (at STP) Mass of water heated = 25 kg Rise in temperature of water between inlet and outlet = 14c Mass of steam condensed = 0.028 kg Find the higher and lower calorific values per m3 at S.T.P. Latent heat of vapourization of water at STP = 2466 KJ/Kg. |  |
| b) | With a neat sketch explain the construction and working of Bomb calorimeter. |  |
| c) | With a neat sketch explain the construction and working of any water tube boiler. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define and represent the following processes on P-V and T-S diagram i)Adiabatic process ii)Isochoric process. |  |
| b) | A quantity of gas has a volume of 0.14 m3, pressure 1.5 bar and a temperature 100c. If the gas is compressed at a constant pressure until its volume becomes 0.112m3, determine: - Temperature at the end of compression - Work done in compressing the gas - Decrease in internal energy - Heat given out by the gas. Cp = 1.005 KJ/Kgk, Cv= 0.712 KJ/Kgk, R=285 J/Kgk. |  |
| c) | With the help of a neat sketch, explain the construction and working of La Mont high pressure boiler. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Steam enters an engine at a pressure of 12 bar with a 67c of superheat. It is exhausted at a pressure of 0.15 bar and 0.95 dry. Find the drop in enthalpy of the steam. |  |
| b) | When is multi-stage compression used for air? Explain the effect of intercooling in a multi-stage reciprocating compressor. |  |
| c) | With the help of a neat sketch, explain the construction and working of blow-off cock. | ….2/- |

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Dry saturated steam at a pressure of 10 bar is expanded isentropically to a pressure of 0.7 bar. Using Mollier diagram, determine the enthalpy drop. |  |
| b) | A steam turbine receives steam at 15 bar and 350c and exhausts to the condenser at 0.06 bar. Determine the Rankine efficiency. |  |
| c) | State the functions of a steam calorimeter. Explain the construction & working of any steam calorimeter with a neat sketch. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Bio-gas energy |  |
| b) | Vane type of air compressor |  |
| c) | Boiler draught |  |
| d) | Contrifugal air compressor |  |
| e) | Limitations of Fossil fuels as energy source |  |

***********