## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-I(4331)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Question No.1 is compulsory. Answer any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Hovercraft |  |
| b) | Bulk carrier |  |
| c) | Form coefficients |  |
| d) | Free surface effect |  |
| e) | Table of off sets |  |
| f) | Centre of bouyancy |  |
| g) | Simpson’s 5,8,-1, rule |  |
| Q.No.2. | A ships waterplane is 80m long. The half breadths commencing from forward are as follows. 0, 3.05, 7.1, 9.4, 10.2, 10.36, 10.3, 10.0, 8.84, 5.75 and 0 mtrs respectively. The spscing between the first three and the last three ordinates is half of that between the other ordinates. Calculate i) Area of waterplane ii) Longitudinal centre of floatation. | (12) |
| Q.No.3. | Answer the following: |  |
| a) | A ship of 8000 tonnes displacement has KM=8.7m, KG=7.6m. The following weights are then added and discharged. Load 250 tonnes cargo KG=6.1m above the keel Load 300 tonnes cargo KG=0.6 m above the keel Discharge 50 tonnes of ballast KG= 1.2 m above keel Find the final KG. | (08) |
| b) | Explain the fairing of lines and the sequence followed in fairing | (04) |
| Q.No.4. | Answer the following: |  |
| a) | Explain with help of neat sketch the various hydrostatic properties of a ship. | (06) |
| b) | A ship is floating on even keel at 6.0m draft forward and aft. The areas of waterplanes are as follows: Draft(m); 0; 1; 2; 3; 4; 5; 6. Area(Sq.m); 5000; 5600; 6020; 6025; 6025; 6020; 6000. Find the ships moulded displacement. | (06) |
| Q.No.5. | Answer the following: |  |
| a) | Draw a neat sketch of GZ curve. Explain various parameters of stability determined from it. | (06) |
| b) | Explain the types of equilibrium in transverse stability. | (06) |
| Q.No.6. | Answer the following: |  |
| a) | A ship displaces 12000 tonne, its centre of gravity is 6.50m above the keel and its centre of buoyancy is 3.60m above the keel. If the second moment of area of waterplane about the centerline is 42.5 x 103 m4, find the metacentric height. | (06) ……2/- |

-2-

| b) | A ship of 5000 tonne displacement has a rectangular double bottom tank 9m wide and 12m long, half full of sea water. Calculate the virtual reduction in GM due to free surface. | (06) |
| --- | --- | --- |
| Q.No.7. | Explain any four of the following hydrostatic properties: | 4 x 3 = 12 |
| a) | MCT |  |
| b) | TPC |  |
| c) | Longitudinal centre of buoyancy |  |
| d) | LCF |  |
| e) | metacentre |  |
| f) | Block co-efficient |  |

***********