## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Civil Engg./CT

Subject: Quantity Surveying & Costing(4014)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the terms (i) Labour contract (ii) Material contract. |  |
| b) | How approximate estimates are prepared for water supply projects? |  |
| c) | State the significance of measurement sheets and abstract sheets in estimating. |  |
| d) | Briefly explain PWD method of taking out quantities. |  |
| e) | Explain the significance of provisional sums and spot items in estimation. |  |
| f) | What are annual maintenance and repair estimates? |  |
| Q.No.2. | Answer any one of the following: | (15) |
| a) | Prepare an estimate for the portion of the road between chainages 1 to 10 for the following data: |  |
|  | Chainage; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10. RL of Ground (m); 151.3; 152.4; 151.9; 152; 149.4; 149.0; 151.7; 151.2; 153.1; 152.1. |  |
|  | i)Formation width of road is 15 m. |  |
|  | ii)Side slopes 2:1 in cutting & banking |  |
|  | iii)Length of one chain = 30 m |  |
|  | iv)Use market rates for cutting and embankment earthwork. |  |
|  | v)Turfing with grass for embankment sides can be taken as Rs.25.00 per square meter. |  |
|  | vi)Assume uniform falling gradient of 1 in 175 passing through ground level at chainage 1. |  |
|  | vii)Draw cross section and longitudinal section also. |  |
| b) | For the Fig. Q. No.2 (b) shown, work out the quantities of any five of the following items: |  |
|  | i)Laterite stone masonry in foundation and plinth in cement mortar 1:5 |  |
|  | ii)PCC 1:3:6 in plinth and below wall footing using 20 mm aggregates. |  |
|  | iii)12 mm internal plaster in cm 1:4 with neeru finish |  |
|  | iv)Flooring with ceramic tiles |  |
|  | v)RCC slab 1:1.5:3, 120mm thick |  |
|  | vi)External plaster 1:3 cement mortar 15mm thick. |  |
| Q.No.3. | Answer any three of the following | 3 x 5 = 15 |
| a) | Explain organizational structure of PWD. |  |
| b) | What are the credentials of a good valuer? |  |
| c) | Explain the importance of task work in Rate analysis. |  |
| d) | Carry out rate analysis of the following item: Laterite stone masonry in cement mortar 1:5. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.4. | Answer any three of the following: | 3 x 5 = 15 |
| a) | Draft a tender notice for inviting tenders for construction of a school building at an estimated cost of Rs.65 lakhs to be completed in 18 months. |  |
| b) | Briefly explain the procedure adopted from inviting of tenders to issuing of work order for construction works. |  |
| c) | What do you understand by Arbitration? Briefly explain salient points of Arbitration and Counciliation Act 1996. |  |
| d) | Briefly explain the terms: i) Cash imprest ii) work Abstract. |  |
| Q.No.5. | Write short notes on any three of the following: | 3 x 5 = 15 |
| a) | Importance of valuation in construction industry |  |
| b) | Types of Engineering contracts |  |
| c) | Legal aspects of specifications |  |
| d) | Relevance of earnest money deposits and security deposits |  |