## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Drives(4115)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the various factors governing the selection of motor for a given drive. |  |
| b) | State at least three advantage of group drive. |  |
| c) | Define stability of motor-load system. Also state the various types of stabilities. |  |
| d) | Which of the electric braking is considered to be most efficient and why? |  |
| e) | State the factors on which rating of heating of motor depends. Define heating time constant & cooling time constant. |  |
| f) | State the type of motor used for following application of electric drives & justify it: i) Lathe ii) Refrigeration & air conditioning. |  |
| g) | Give two points of distinction between limit switch & float switch. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw the joint speed torque characteristics of electric motor & driven unit and explain it in brief. What is its significance? |  |
| b)i) | State the basic torque equation of motor driven load system. Name each term used in equation. | (2) |
| ii) | Also state the condition to be satisfied for attaining steady state stability. | (2) |
| iii) | Draw a graph of variation of load torque w.r.t. angular displacement of shaft of rotor of motor. Also label the components of load torque. | (2) |
| c) | A horizontal conveyor belt moving at a uniform speed of 1.2 m/sec transports material at a rate of 100 tonnes/hour. Belt is 200 m long & driven by motor at 1200 r.p.m. |  |
|  | i)Determine the load inertia referred to the motor shaft |  |
|  | ii)Calculate the torque that motor should develop to accelerate the belt from standstill to full speed in 8 seconds. Moment on inertia of motor is 0.1 kg-m2. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | State the types of electric braking applicable to d.c. shunt motor. Explain any one of such braking of your choice in brief with the help of neat circuit diagram. |  |
| b)i) | Is regenerative braking applicable for d.c series motor? Give your choice & justify it. | (2) |
| ii) | Draw the speed v/s torque characteristics of plugging in d.c. series motor and explain it in brief. | (4) |

………2/-

-2-

| c)i) | State the conditions to be satisfied for occurrence of regenerative braking in 3 phase Induction motor. |  |
| --- | --- | --- |
| ii) | Under which processes do such conditions occur in 3 phase induction motor. |  |
| iii) | Draw speed v/s torque characteristics of regenerative braking in 3 phase Induction motor. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | How is it possible to obtain speed control in 3 phase squirrel cage induction motor by using Pole changing method. Hence explain the speed control for four pole changing with the help of neat connection diagram. Also draw the connection diagram for constant torque operation in pole changing for stator. |  |
| b) | How is it possible to control the speed of 3 phase slip ring induction motor by using cascade connection. Hence explain in brief with the help of neat circuit diagram. |  |
| c) | What is slip power? How can it be recovered? Hence draw a circuit diagram to show slip power recovery control and explain it in brief. |  |
| d) | What is the effect of load inertia on the rating of drive? Explain it in brief. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Enlist the various classes of duty. Explain any one of such classes of your choice with the help of neat load diagrams such as load v/s time, electric losses v/s time & temperature v/s time cures. |  |
| b) | What is the necessity of load equalisation? How can it be achieved? Explain in brief with the help of speed v/s time curve & torque v/s time curve. |  |
| c) | A load cycle of a motor for 15 minutes in driving following loads is as follows: |  |
|  | 1. 0-5 minutes – 30 H.P. |  |
|  | 2. 5-9 minutes – No load |  |
|  | 3. 9-12 minutes – 45 H.P. |  |
|  | 4.12-14 minutes – No load |  |
|  | Load cycle is repeated indefinitely. Suggest a suitable size of continuously running motor to suit the above conditions. |  |
| d) | Write a short note on drive motor used in sugar mill or textile mills. |  |
| Q.No.6. | Answer any three of the following: | 3 x 4 = 12 |
| a) | State the functions of following switches: |  |
|  | 1) Time delay relay 2) Overload relay 3)Contactor. Also give one point of distinction between contactor & relay. |  |
| b) | Draw the control circuit diagram showing auxillary contact interlocking operation of motor & explain it in brief. |  |
| c) | Draw the control circuit diagram showing direct method of reversal of direction of rotation of motor & explain it in brief. |  |
| d) | Draw the control circuit diagram to show remote control operation of an induction motor & explain it in brief. |  |

***********