## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electrical Engg./E&EE

Subject: Switchgear & Protection(4110)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State advantages of per unit system for fault calculation. |  |
| b) | List the advantages of having reactor control of short circuit currents. |  |
| c) | What is a fuse? For a fuse define i) fusing current ii) fusing factor. |  |
| d) | Define descrimination. Explain clearly with the help of 2 fuses. |  |
| e) | Explain relay circuit connection. |  |
| f) | Discuss the role of backup protection. |  |
| g) | Explain the following terms as applied to circuit breakers i) Restriking voltage ii) Recovery voltage. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram, explain Plain break oil circuit breaker. State its disadvantages. |  |
| b) | Explain ELCB with a neat diagram. |  |
| c) | State the methods to be adopted for arc extinction in A.C. circuit breakers & explain each method in brief. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give constructional features of MOCB. |  |
| b) | Explain the working of HVDC circuit breaker. |  |
| c) | With a neat diagram describe Sf6 circuit breaker. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain electromagnetic attraction relays in brief. |  |
| b) | Describe in brief the construction & working of structures commonly used for obtaining the phase difference in fluxes in induction relays. |  |
| c) | With a neat block diagram, explain the working of static overcurrent relay. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain differential protection scheme for alternators with a neat diagram. |  |
| b) | Describe Distance protection scheme for high voltage transmission lines. |  |
| c) | Explain the time graded scheme of protection parallel & ring feeder. |  |
| Q.No.6. | Write short notes on any two: | 2 x 6 = 12 |
| a) | Carrier current protection scheme |  |
| b) | Bus bar protection |  |
| c) | Translay scheme for transmission line |  |

***********