## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Common

Subject: Non Conventional Sources of Energy(3009)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain role of renewable energy sources in the present energy situation. |  |
| b) | What is the need for sun tracking in focusing collectors? |  |
| c) | Give any three merits of a solar cooker. |  |
| d) | State any three advantages of a floating drum biogas plant. |  |
| e) | State any three limitations of wind energy systems. |  |
| f) | What are geopressured systems? |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) i) | Why is there a need for storage of solar energy? | (2) |
| ii) | Explain with a sketch the pumped hydroelectric storage. | (4) |
| b) | Explain the parabolic trough line focusing collector with a neat sketch. |  |
| c) | Describe the working of a typical flat plate liquid collector with the help of a sketch. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) i) | With the help of simple sketches explain briefly the difference between porous and non-porous type air heaters. | (4) |
| ii) | State any two applications of solar air heaters. | (2) |
| b) | Explain the working of a solar pond electric power plant with the help of a figure. |  |
| c) | With reference to passive solar heating, explain with the help of suitable sketches: i) Thermal storage wall ii) Convective loop | (3+3) |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) i) | What is the principle of solar cell? | (3) |
| ii) | State any four advantages of solar cell. | (3) |
| b) | Explain briefly the following biomass conversion technologies: i) Thermo chemical conversion ii) Biochemical conversion. | (3+3) |
| c) | Discuss any four factors affecting biogas generation. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the classification of a wind energy conversion system according to i) Axis of shaft ii) Size iii) Ultilisation of output iv) Output power. |  |
| b) | Explain with the help of a figure the wind energy system with battery storage. |  |
| c) | Discuss various effects of wind turbines on the environment. |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain a double basin tidal system with the help of a sketch. |  |
| b) | With the help of a sketch, explain briefly the typical arrangement of a small hydropower scheme. |  |
| c) i) | State the principle of ocean thermal energy conversion system. | (2) |
| ii) | Give any four advantages of geothermal energy over other forms of energy. | (4) |

***********