## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Theory of Structures-I(4022)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Define shear connectors and explain how will you find spacing of the same. |  |
| b) | Differentiate between Determinate and Indeterminate structures with atleast 2 examples. |  |
| c) | Derive expression for limit of eccentricity for a circular column for no tensile stress condition. |  |
| d) | A solid circular shaft of 6 mm diameter rotates at 120 rpm. Determine power transmitted if shear stress is 45 N/mm2 for material of shaft. Take maximum torque 30% more than average torque. |  |
| e) | Draw Influence diagram for shear force due to rolling load along with analytical solution. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A timber beam 120mm wide and 200mm deep is reinforced by a steel plate of 10mm thick and 150mm deep connected to both sides of timber beam. If stress in wood is 6N/m2. Find moment of resistance of beam. Take ES= 15EW. |  |
| b) | A wooden beam of 150mm wide and 250mm deep consists of a steel plate of 10mm thick and 150mm wide connected to top and bottom of wooden beam. If permissible stresses are 6N/m2 and 60N/mm2 in wood and steel respectively. Find moment of resistance of beam. Take ES= 16EW. |  |
| c) | Find degree of indeterminacy of following structures: i) A King post tress ii) A fixed beam with a central hinge iii) Three hinged arch. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define Kern of a section and draw the same for i)Rectangular column ii)Circular Column iii)Hollow circular column with limit of eccentricity. |  |
| b) | A hollow circular column has outer diameter 220mm and thickness 25mm. A vertical load of 500 KN acts at a distance of 80mm from axis of column. Find maximum and minimum stresses in the column. |  |
| c) | A masonry chimney 18 meters high is of circular section of outer diameter 5m and internal diameter 3m. The chimney is subjected to horizontal wind pressure of 1500N/m2 of projected area. Find maximum and minimum stresses at the base. Take density of masonry as 21KN/m3. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | At a point in a beam there is a longitudinal bending stress of 120N/mm2 and a transeverse shear stress of 50N/mm2. Find resultant stress on a plane inclined at 30º to the longitudinal axis. |  |

………2/-

-2-

| b) | At a point in a material normal stresses of 1000/mm2 and 60mm2 both tensile acts on two planes at right angles to each other along with shear stress of 40N/mm2 across both planes. Determine magnitude and direction of principal stresses and maximum shear stress. |  |
| --- | --- | --- |
| c) | An Elastic material is subjected to direct stresses of 60N/mm2 and 30N/mm2 both tensile in two mutually perpendicular planes. Using mohr’s circle of stress locate the plane across which resultant stress has a maximum obliquity. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A simply supported girder of 12m span has a UDL of 25KN/m of 4 m length rolling from left end. Calculate the maximum bending moment at a section of 8m from left support using ILD concept. |  |
| b) | Two wheel loads of 60KN and 80KN spaced 1.2m apart with 80KN leading load is moving on the girder of span 15m. Determine maximum positive and negative shear force in girder at a section of 6m from left support. Use ILD concept. |  |
| c) | A simply supported beam of 10m span carry a point load of 20KN at 2m from left and a UDL of 40KN/m for right half span. Determine reaction at supports using concept of ILD. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Solid circular shaft rotates at 90 rpm transmitting power of 75KW. Determine suitable diameter of shaft if shear stress is not to exceed 50N/mm2 and angle of twist not to exceed 2º (degree) in a length of 3m shaft. Take rigidity modulus of material of shaft as 0.80x105 N/mm2. Maximum torque exceed mean torque by 35%. |  |
| b) | A thin tube 60mm diameter and 4mm thick is subjected to a torque of 1000Nm. Find shear stress in tube and angle of twist in a length of 600mm. Take rigidity modulus as 80KN/mm2. |  |
| c) | - State the assumptions made in theory of pure torsion. - Define polas modulus and derive expression for same for solid circular shaft and a hollow circular shaft. | (2) (4) |

***********