## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Mechanical Engg.

Subject: Fluid Machinery(4040)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | A jet of 50 mm diameter strikes on series of plates with 20m/s. The plates move with a velocity of 10m/s. Determine the work done per second on the plate. |  |
| b) | Draw the symbols for following components of Hydraulic system. - 4/3 lever operated DC valve - Reversible hydraulic pump. |  |
| c) | Why priming is necessary in centrifugal pump? |  |
| d) | Draw symbols for the following pneumatic components: (i) Shuttle valve (ii) Non-Return Valve. |  |
| e) | Draw & label parts of an unbalanced vane pump. |  |
| f) | What is an air vessel? Explain its use on suction & delivery side of Reciprocating pump. |  |
| g) | Define specific speed of turbine. What is its significance? |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain governing of an Impulse turbine. |  |
| b) | Differentiate between impulse turbine and reaction turbine. |  |
| c) | A Pelton wheel, having semi-circular buckets and working under a head of 140 metres, is running at 600 rpm. The discharge through nozzle is 500 litres/sec and diameter of wheel is 600 mm. Find (i) Power available at the nozzle (ii)Hydraulic efficiency of the wheel. Assume Cv = 0.96. |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| a) | A centrifugal pump of 1.5 metre diameter runs at 210 rpm and pumps 180 litres/sec of water. The angle which vane makes, at exit, with the tangent to the impeller is 25. Assuming radial entry and velocity of flow throughout as 2.5 m/s, determine the power required to drive the pump. If manometric efficiency of the pump is 65 per cent, find average lift of the pump. |  |
| b) | Describe any two types of casing with neat sketches. |  |
| c) | Explain multistage centrifugal pumps. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| a) | A single acting reciprocating pump, having plunger diameter 125 mm and stroke length 300mm is drawing water from a depth of 4 metres from the axis the cylinder at 24 rpm. The length and diameter of suction pipe is 9 metres and 75 mm respectively. Find the pressure head on the piston at the beginning and end of the suction stroke, if the barometer reads 10.3m of water. |  |

….2/-

-2-

| b) | A single acting reciprocating pump (with no air vessel) has a plunger of 80 mm diameter and a stroke of 150 mm. It draws water from a sump 3m below the pump axis through a suction pipe 30 mm diameter and 4.5 m long. If separation occurs at a pressure of 80KPa below atmospheric pressure, find maximum speed at which the pump may be operated without separation. Assume that the plunger moves with simple harmonic motion. |  |
| --- | --- | --- |
| c) | Show from first principles, that the work saved against friction in the delivery pipe of double acting reciprocating pump, by fitting air vessel, is 39.2%. |  |
| Q.No.5. | Answer any two: | 2 x 6 = 12 |
| a) | A francis 135 KW turbine, having overall efficiency of 75% is required to produce a head of 9 metres and at 120 rpm. The velocity of periphery of the wheel and velocity of flow at inlet 3.47Hand 1.1Hrespectively. If hydraulic losses in the turbine are 20% fo the available energy, find: - guide blade angle at inlet, - wheel vane angle at inlet, and - diameter of the wheel. |  |
| b) | A Kaplan turbine , operating under a net head of 20 metres, develops 20,000 KW with an overall efficiency of 66 percent. The speed ratio is 2.0 and flow ratio is 0.6. The hub diameter of the wheel is 0.35 times the outside diameter of wheel. Find diameter and speed of turbine. |  |
| c) | Explain and compare the performance of different turbines with the help of operating characteristic curves. |  |
| Q.No.6. | Answer any two: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the working of an External Gear pump. |  |
| b) | Explain the working of a Counter balance circuit in a hydraulic system. |  |
| c) | Explain the working of a flow control valve used in pneumatic system with the help of a neat sketch. Write the symbol for the same. |  |

***********