## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electrical Engg.

Subject: Energy Management(5108)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define i)Demand factor ii) Load factor. |  |
| b) | What are the advantages of power factor improvement? |  |
| c) | Explain the need for Energy Audit. |  |
| d) | Compare conventional and Electronic ballast. |  |
| e) | Explain briefly the determination of motor loading by any one method. |  |
| f) | Define pump capacity and hydraulic power. |  |
| g) | Explain the concept of Load side energy management. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the quality of incoming supply and its effect on the performance of machinery. |  |
| b) | List down the features of Energy conservation act 2001 and also explain the importance of energy conservation. |  |
| c) | Briefly explain the energy audit measuring instruments. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the step by step approach for maximum demand control. |  |
| b) | A factory takes a load of 200 KW at 0.85 pf lagging for 2500 hours per annum. The tariff is Rs.150 per KVA plus 5 paise per KWh consumed. If the p.f. is improved to 0.9 lagging by means of capacitors costing Rs.420 per KVAR and having a power loss of 100W per KVA, calculate the annual saving effected by their use. Allow 10% per annum for interest and depreciation. |  |
| c) | A single phase motor connected to a 240V, 50Hz supply takes 20A at a power factor of 0.75 lagging. A capacitor is shunted across the motor terminals to improve the p.f. to 0.9. Determine the capacitance of the capacitor to be used. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the procedure for assessment of lighting systems. |  |
| b) | How is pump efficiency determined? Briefly explain the procedure. |  |
| c) | Define and explain the terms i) colour rendering index ii) installed load efficiency iii) Average maintained luminance. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain any two efficient lighting controls used. |  |
| b) | Write a short note on Automatic power factor controllers. |  |
| c) | Explain the concept of variable frequency drive. |  |

………2/-

-2-

| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Technical aspects of Energy efficient motors |  |
| b) | Soft starters and its advantages |  |
| c) | Objectives of tariff and desirable characteristics of a tariff |  |
| d) | Importance of flattening of load curve |  |

***********