## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Engineering & Technology

Subject: Applied Mechanics-II(2006)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | In a lifting machine, an effort of 1.5 KN is required to move 24KN. What is the VR of this ideal machine? |  |
| b) | Define perfect & imperfect frame. |  |
| c) | A projectile is projected at 30 with an initial velocity of 30 m/s. Find its velocity when it reaches the maximum height. |  |
| d) | Represent work done by a spring & a constant force, graphically. |  |
| e) | State & explain D’Alembert’s principle with a neat sketch. |  |
| f) | Determine angular retardation if the power supply is cut off to a power driven wheel rotation at 600rpm if it has made 200 revolutions before coming to rest. |  |
| g) | Define impulse & momentum. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive an expression for VR of a single threaded worm & worm wheel, with neat sketch. |  |
| b) | Determine the VR of a machine if it is found that in one trial an effort of 120N is required to lift a load of 4.2 KN & in another trial an effort of 150N is required to lift 11.4 KN. Also establish law of this machine. Calculate efficiency at 150N effort. It is a single purchase crab having 220mm diameter of load drum, 1m length of lever arm & number of teeth on pinion & spur are 10 & 120 respectively. |  |
| c) | The following particulars are given to a screw jack: Diameter of screw jack= 62.5mm, length of handle =250m pitch of threads = 12.5 mm, coefficient of friction =0.05. Find the effort required to lift 5 KN load. Also state if the machine is reversible & why? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A fire brigade man wants to extinguish fire at a height of 6m above the nozzle standing at a distance 5m away from the fire. Find i) The minimum velocity of the nozzle discharge required ii)Velocity of discharge if he could extinguish the fire with an angle of projection of 60. |  |
| b) | Maximum range of a field gun is 2000m. If a target at a distance of 1200m is to be hit, what should be the angle of projection? |  |
| c) | An electric motor runs at 1200 rpm & does a work of 240 KJ per minute. Find the torque on the motor shaft. Also find the work done when the shaft turns through 60. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A person standing on a stationary boat fires a shot at a target on the bank. The mass of the bullet is 10gms & its muzzle velocity is 900m/s, horizontal. Find the velocity with which the boat moves back if mass of boat with the person is 300kgs. Neglect water resistance. |  |

………2/-

-2-

| b) | A truck of mass 5000 Kgs ascends up an inclined plane of 2% by a constant engine pull of 1050N. If the resistance to motion is 30N/t mass, & initial velocity is 6 m/s, how far up will the truck go before velocity is reduced to 3 m/s. |  |
| --- | --- | --- |
| c) | A pump reduces the water level in a well, 4m diameter by 2 metres in 20 minutes and lifts through an average height of 5 m. If the efficiency of the pump is 80%, what power is required to drive the pump? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A wire rope supports a mass of 1800 Kgs. What is the tension in the rope if the lift is i)Moving up with an acceleration of 0.8 m/s2 ii)Moving down with an acceleration of 0.8 m/s2 iii)Moving with uniform velocity. |  |
| b) | In what distance will body A shown in FIG.(1) attain a velocity of 3 m/s, starting from rest. |  |
| c) | Find values of P (the horizontal force) & tension T1 & T2 in the strings in FIG.2. Mass of each block is 30 Kgs. =0.2 & acceleration of the system is 0.18 m/s2. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Analyse the force in members BD,BC,AC,CD & CE roof truss shown in Fig (3). Use method of sections. |  |
| b) | Analyse the cantilever truss shown in fig.(4). |  |
| c) | Analyse roof truss shown in fig (3), completely using method of joints. | …3/- |