## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Automobile Engg.

Subject: Design & Drawing of Automobile Components(4067)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw stress-strain diagram for ductile material. How will you determine i) Modulus of elasticity ii)Percentage elongation from them. |  |
| b) | Distinguish between “Brittleness” & “Ductility”. |  |
| c) | Prove that square key is equally strong in crushing and shearing. State the assumptions you make. |  |
| d) | State the function of couplings and clutches. Why does a cone clutch transmit more power than a plate clutch? |  |
| e) | What is the difference between forms of threads used for fastening and those used for power screw? |  |
| f) | What do you mean by a column? What is the effect of end conditions on the crippling load capacity of a column? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the commonly used spring materials? What are the factors which govern the choice of a particular material? |  |
| b) | A close coiled helical compression spring of 16 active coils having a spring stiffness of 10Kg/mm is cut into two springs having 4 and 12 turns. Determine the spring stiffness of resulting springs. If it is cut into two equal halves, what will be the new value of spring stiffness? |  |
| c) | A semi elliptical laminated spring 900mm long and 55mm wide is held together at the centre by a band 50mm wide. If the thickness of each leaf is 5mm, find the number of leaves required to carry a load of 4500N. Assume working stress of 490N/mm2. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A line shaft transmits 25KW power at 200rpm by means of a vertical belt drive. The diameter of the belt pulley is 1m and the pulley overhangs 150mm beyond the centre line of the end bearing. The belt tensions act vertically downward. The tension on the tightside of the belt is 2.5 times that on slack side. The shaft is made of plain carbon steel 40C8 (Syt=380 N/mm2) and the factor of safety is 2.5. The mass of the pulley is 25kg. Determine the diameter of the shaft. Ref. Fig.1. |  |
| b) | A bracket made of cast iron FG200 is subjected to a load of 5KN as shown in fig.2. Determine the required thickness ‘t’ at the section xx, if the factor of safety is 4. |  |
| c) | The shaft of an overhang crank is subjected to a force F of 1.5 KN as shown in fig.3. The shaft is made of 30Mn2 steel having the allowable shear strength equal to 80N/mm2. Determine the diameter of the shaft. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The structural connection shown in fig.4 is subjected to an eccentric force ‘P’ of 10KN with an eccentricity of 500mm. The centre distance between bolts 1 and 2 is 200mm and the centre distance between 1 and 3 is 150mm. All the bolts are identical. The bolts are made from plain carbon steel 30C8 (Syt=400 N/mm2) and the factor of safety is 2.5. Determine the size of the bolts. |  |
| b) | The standard cross-section for a flat key, which is fitted on a 50mm diameter shaft, is 16x10mm. The key is transmitting 475 N-m torque from the shaft to the hub. The key is made of commercial steel (Syt=Syc=230 N/mm2). Determine the length of the key if the factor of safety is 3. |  |
| c) | A rigid flange coupling is required to transmit 20KW power at 720 rpm. The shafts and bolts are made of plain carbon steel 30C8 (Syt=400N/mm2) and the factor of safety is 4. Assume Syc=1.5 Syt and Ssy = 0.5 Syt. Determine - Diameter of shafts - Dimensions of flanges - Diameter of bolts. |  |

………3/-

-3-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Design the pin diameter of a bush pin type flexible coupling for connecting a motor shaft to a pump shaft for the following service conditions. Power to be transmitted = 40KW R.P.M of motor shaft = 1000 Diameter of motor shaft = 50mm Diameter of pump shaft = 45mm The bearing pressure in the rubber bush is 0.35 N/mm2 Given Data Allowable stress in the pins is 25 N/mm2 Power to be transmitted = 40 KW = 40 KNm/s R.P.M of motor shaft, N = 100 Diameter of motor shaft (d) = 50mm Diameter of pump shaft = 45mm Bearing pressure in the bush (Pb= 0.35 N/mm2) Allowable stress in the pins = 25N/mm2. |  |
| b) | A multi-disc clutch employs 3 steel and 2 bronze discs having outer diameter 300mm and inner diameter 200mm. If the coefficient of friction is assumed as 0.22, find the axial pressure and the horse power transmitted if the unit normal pressure is 0.13 N/mm2 and RPM is 750. |  |
| c) | Determine the diameters of the hollow push rod made of mild steel for an I.C engine if the maximum force exerted by the push rod is 1500N. The length of the push rod is 50cms. Take factor of safety as 2.5 and the end fixity coefficient as 2. Take E= 210 KN/mm2. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | - Explain with a neat sketch the difference between tolerance and Allowance. - Sketch the different types of fits used in engineering practice. |  |
| b) | Define factor of safety. List the important factors that influence the selection of appropriate factor of safety |  |
| b) | Following are the specifications for a four-stroke petrol engine Piston diameter = 120mm Stroke = 140mm Maximum speed = 40 rev/sec Maximum explosion pressure = 2.45 MPa Length of the connecting rod from center to center = 315mm Weight of the reciprocating parts = 18N Design the diameter of the two big end bolts and the thickness of the cap. Given that the distance between the centers of the two bolts as 62mm and length of the big end bearings as 52mm. Take allowable tensile stress for both materials as 120 MPa. |  |
|  | -4- |  |
| c) | Design the crown thickness of a C.I piston for a single acting 4 stroke engine for the following specification. Cylinder bore = 102mm Stroke = 120mm Max. gas pressure = 4.8 MPa Brake mean effective pressure = 0.6 MPa Speed = 2200 rpm Fuel consumption = 225gm/KWh Allowable tensile stress for C.I = 37 MPa Temperature difference between centre and edge of crown=220c Heat conductivity factor = 47w/m/oc HCV of fuel = 41880 KJ/kg |  |

…..5/-