## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Process-I(4233)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Distinguish clearly between accuracy and precision. |  |
| b) | Explain how will you mark holes in tee section. |  |
| c) | Explain the methods of marking out bolt holes for flanges. |  |
| d) | State the requirements of good dial indicators. |  |
| e) | What is standard of measurements? Explain line standard and end standard. |  |
| f) | What is the correct procedure to extinguish the flame in flame cutting? |  |
| g) | What safety precautions are to be taken in metal sawing to prevent accidents? |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain with a neat sketch working of a reciprocating power hacksaw and state its applications. |  |
| b) | Give the basic working principle of flame cutting. Enumerate the various factors influencing the quality of the cut. |  |
| c) | Write the application-wise material selection for templates and the reasons for selecting a particular material. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain how the correct size of cutting nozzle to be used in oxygen fuel gas cutting process is determined. |  |
| b) | With the help of diagrammatic representations, explain the different types of limits and fits. |  |
| c) | Explain with a neat sketch working principle of an Universal level protractor or Vernier protractor. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain with the help of neat sketches the following terms: i) Basic size ii) Limits of size iii)Deviations iv) Zeroline v)Tolerance vi)Fundamental deviation. |  |
| b) | Explain in brief how straightness is tested by using spirit level. |  |
| c) | What are limit gauges? Explain with sketches: i) Plain plug gauges ii) Snap gauges and iii) Contour gauges. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Compare the method of direct marking and use of templates for marking and give their advantages and disadvantages. |  |
| b) | Discuss on materials used for templates, protection and storage of templates. |  |
| c) | Explain the procedure involved to mark the location of 4 number of holes on a flange at a p.c.d of 100 mm. |  |
| Q.No.6. | Write short notes on any four of the following: | 4 x 3 = 12 |
| a) | Nozzle mix cutting torch b) Flatness testing |  |
| c) | Abrasive cutting disc d) Marking of holes in column or beams |  |
| e) | Dividers and compasses f) Requirements of good micrometer |  |

***********