## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electrical Engg.

Subject: Distribution & Estimation(4108)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.I. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| 1) | Switch and fuse are connected in live wire and not in neutral wire. Why? |  |
| 2) | State the classification of distribution system in detail. |  |
| 3) | Briefly discuss planning of construction work. |  |
| 4) | State the important points to be considered for installation of motor. |  |
| 5) | Define the term diversity factor and demand factor. Give their significance. |  |
| 6) | State the factors influencing earth resistance. |  |
| Q.No.II. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | Find the inductance per km of 3 phase transmission line using 1.24 cm diameter conductor when these are placed at the corner of an equilateral triangle of each side of 2 mtrs. |  |
| 2) | Explain different types of faults occurring on power system. Also, discuss on frequency of faults occurrence on different items of power system. |  |
| 3) | Compare radial, ring and interconnected system on the basis of load, reliability of supply and economy. |  |
| Q.No.III. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | Write short note on Rural Electrification. |  |
| 2) | Prepare bill of material for 1 Km of 11 KV line. (100 mtr span) |  |
| 3) | A 11KV/400V /Y connected distribution transformer is to be erected in village to meet the following loads: - One workshop with 10 KW load operating at 0.8 p.f. - Two agricultural pumps one of 3HP and another of 5HP. - 30 houses whose lighting load is 96 KW. Select a suitable capacity transformer and calculate its primary and secondary current. |  |
| Q.No.IV. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | Draw a neat sketch of Guy assembly. Label its parts and state their functions. Also explain different types of Guys and Guying arrangement. |  |
| 2) | Draw a neat sketch of overhead 11 KV line. Name its parts and also state function of guard wire, cross arm and earth wire. |  |
| 3) | State the points to be considered at the time of erection of overhead lines. |  |

………2/-

-2-

| Q.No.V. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| 1) | What is the need of earthing? With a neat sketch, explain plate earthing. |  |
| 2) | Explain conduit wiring system and state its advantages and disadvantages. |  |
| 3) | Explain the tests to be performed on wiring installation. (any two) |  |
| Q.No.VI. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | Write short note on ELCB. |  |
| 2) | Draw and explain circuit diagram for: i)Three lamp and one socket outlet controlled by individual switches ii) Godown wiring. |  |
| 3) | In a small workshop of size 30m x 15 m, one number 15 KW, 400V, 3, 50 Hz motor is to be installed. Draw wiring plan, single line and wiring diagram for the same. Calculate the rating of main switch. |  |

***********