## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Applied Electronics(4213)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.I. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| 1) | Define any 3 hybrid parameters of a transistor. |  |
| 2) | Explain the concept of cascading amplifier stages. Write the expression for overall voltage gain in dB’s of a cascaded amplifier. |  |
| 3) | Compare power dissipation in a Class A amplifier with no i/p signal and with maximum input (i/p) signal. |  |
| 4) | What is the Barkhausens criterion for sustained oscillations? |  |
| 5) | Write a note on Bandwidth of FM waves. |  |
| 6) | Draw the circuit that will perform the following operation: Vo= -V1 –V2 – V3 . |  |
| Q.No.II. | Answer any three of the following: | 3 x 4 = 12 |
| 1) | Draw the characteristics of a CE transistor and obtain any two h-parameters from the same. |  |
| 2) | State the advantages of expressing gain in dB. Also explain the circuit for direct coupled amplifier. |  |
| 3) | Describe with waveforms the following amplifier classes: i) Class B and ii) Class C. |  |
| 4) | Define CMRR. Why is CMRR infinite if a true constant current source is used in a symmetrical emitter coupled differential amplifier. |  |
| Q.No.III. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | In a negative feedback amplifier, A=100, =0.04 and Vi= 50mv. Find - Gain with feedback - Output Voltage - Feedback factor - Feedback Voltage. |  |
| 2) | Draw the BJT circuit employing voltage shunt feedback and explain its working. How is feedback factor defined for this circuit? |  |
| 3) | Explain one application of each of the following: - Differential amplifier - Transformer Coupled amplifier - Power amplifier. |  |
| Q.No.IV. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | Explain amplitude modulation using waveforms, state its advantages and also derive the power relations. |  |
| 2) | What is SSB modulation? State its advantages. Calculate the percentage power saving when carrier and one sideband is suppressed in AM to a depth of 50%. |  |
| 3) | Explain the generation of FM using basic reactance modulator. |  |

………2/-

-2-

| Q.No.V. | Answer the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| 1) | Using neat circuits, explain the operation and state expression for output of the following: - Op – amp Integerator - Op – amp Differentiator. |  |
| 2) | Draw the internal block diagram of a typical Operational amplifier. Give the electrical parameters and specifications of a 741 IC op – amp. |  |
| Q.No.VI. | Answer any three of the following Questions: | 3 x 4 = 12 |
| 1) | Draw and explain the working of Band stop filter. |  |
| 2) | Draw a circuit using operational amplifier such that it can be used to amplify charge and explain its working. |  |
| 3) | Calculate the value of voltage gain, current gain, input impedance and o/p impedance for a transistor amplifier having following specifications: hie = 6.4k, hfe = 270, hre = 1.5 x 10-4, hoe = 6mhos, Rs = 1k, RL = 10k. |  |
| 4) | Write a note on: - Two Port transistor model - Frequency Spectrum for AM. |  |

***********