## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electrical Engg./E&EE

Subject: Substation Practice(5105)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain in brief the points to be considered in selecting the site for a substation. |  |
| b) | Why sectionalisation of Bus-bar in done? |  |
| c) | Explain the term ‘Burden’ as related to CT’s and PT’s. |  |
| d) | What is the sequence of operation while opening & closing of Isolators, circuit breaker & earthing switches? |  |
| e) | What is a Semaphor Indicator provided control panel? What do you mean by mimic diagram? |  |
| f) | What are the different types of control & Relay Panel? |  |
| g) | Give comparison between power and control cables. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the classification of substation according to constructional features. |  |
| b) | With a neat sketch explain the working of Double Bus Bar arrangement. |  |
| c) | List out the various components of a substation with their symbols. Draw a single line diagram of pole-mounted substation. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The secondary of a CT should not be kept open circuited. Give reason. State the specifications for procurement of CT’s & PT’s. |  |
| b) | What is the difference between system earthing & equipment earthing? State advantages of neutral earthing. |  |
| c) | List the different methods of earthing used in a substation. Explain any two methods. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are harmful effects of lightning? How do earthing screen & ground wires provide protection against direct lightning strokes? |  |
| b) | What is Tariff? Explain any one in detail giving its merits & demerits. |  |
| c) | Discuss the various methods of power factor improvement. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain general construction of power cable. |  |
| b) | Describe the method of laying power cable in trenches below ground level with a neat sketch. State its merits & demerits. |  |
| c) | Write a short note on Traction substation. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a short note on Line clearance and safety precautions in carrying out work in substation. |  |
| b) | Explain the methods of charging lead acid batteries. |  |
| c) | Explain how dielectric strength of oil is tested. |  |

*************

-2-

***********